Design and Implementation of a PID Controlled Human Following Robot

Arif  Islam			Mahir Faisal Ahammed		Mustarashid Ahmed Aurvo
2222651643			2211953043			2211080043

Abstract-This paper describes the design of a wirelessly connected human-following PID controlled robot. A laptop camera and Python, OpenCV and MediaPipe libraries are used to detect and estimate the position of a human. Communication between the laptop and robot is managed by an ESP32 microcontroller using WiFi. An Arduino Uno receives commands and drives four DC motors using an L298N motor driver. Smooth and proportional motion is ensured using a PID controller. The robot effectively follows a human with dynamic speed, distance and direction adjustments.

I. INTRODUCTION

Human following robots can be used in aged care, industrial automation, transportation and security. The capability of a robot to automatically identify and track a human without human intervention is a significant step forward in the field of autonomous robots. Our project seeks to develop a low cost human following robot using off the shelf components and open source software.

Our system uses a laptop webcam for vision, and Python and MediaPipe for human body detection in real-time. The position is used to calculate the desired direction for the robot which is wirelessly communicated to the robot using WiFi to an ESP32 microcontroller. The ESP32 passes on commands to an Arduino Uno connected to the robot motors through an L298N motor driver. A PID controller is used to achieve proportional following.

The goals of this project are: (1) to achieve real-time human detection using computer vision, (2) to set up wireless data transfers between human detection system and the robot, and (3) to implement a PID controller for smooth autonomous following.

II. SYSTEM ARCHITECTURE

The system is comprised of two subsystems: the software detection system on a laptop computer and the hardware control system on the robot chassis, with wireless communication using WiFi socket protocol (illustrated in Fig. 1).

- Software Subsystem

The software subsystem is implemented on a laptop with three components. OpenCV processes the video stream from the camera. MediaPipe is used to detect human body landmarks in real-time, with 33 landmarks detected such as nose and shoulders. The PID controller generates motor speed commands proportional to the error from the camera frame centre. The commands are sent to the ESP32 using Python socket programming.

B. Hardware Subsystem

The hardware subsystem includes an ESP32 microcontroller to receive WiFi signals, an Arduino Uno for motor control, an L298N dual H-bridge motor driver and four-wheel differential drive. The motor driver is powered by two 18650 lithium-ion batteries (7.4V) and the ESP32 is powered by a USB power bank for stability.

Laptop (Python + OpenCV + MediaPipe)

↓ WiFi (TCP/IP)

ESP32 (WiFi Server, Port 8888)

↓ UART (GPIO 16, 17)

Arduino Uno (Motor Logic)

↓ PWM Signals (Pin 5,6,7,8,9,10)

L298N Motor Driver

↓

4-Wheel DC Motor Chassis

Fig. 1. System Architecture Block Diagram

III. HUMAN DETECTION ALGORITHM

We detect humans using Google MediaPipe Pose library which detects 33 landmarks. The landmarks are used to calculate two metrics.

A. Directional Error

The nose landmark (landmark 0) is used to detect the lateral position of the person. The error is the difference between the nose x-coordinate and the middle of the camera frame (320 pixels in a 640-pixel wide frame). If the error is positive then the person is on the right, if it is negative the person is on the left.

Error_x = cx − 320

B. Distance Estimation

The width of the person's shoulders - the horizontal pixel distance between the left shoulder (landmark 11) and right shoulder (landmark 12) - is used to estimate the person's distance from the camera. The greater the width, the closer the person. The following distance is considered safe if it is between 200 pixels (too close) and 100 pixels (too far).

IV. PID CONTROLLER DESIGN

In Python, a PID (Proportional-Integral-Derivative) controller is designed to calculate the smooth speed commands for the motor, based on the position error. The equation for a PID controller is:

u(t) = KP·e(t) + KI·∫e(t)dt + KD·de(t)/dt

The proportional gain (KP = 0.5) corrects the error instantly by a factor of the current error. The integral term (KI = 0.01) sums the error to correct a steady-state small error (integral is clamped between [−100, 100] to avoid windup). The derivative term (KD = 0.1) helps to smooth oscillations by reacting to the change in error.

The PID signal is then used to modulate the motor speed between 80 and 255 (PWM values generated by Arduino), enabling proportional correction of the motor speed.

V. COMMUNICATION SYSTEM

The robot and the laptop communicate wirelessly via TCP/IP socket programming with WiFi. The laptop Python code is a client that connects to the ESP32 server on port 8888.

The commands are sent as a string of characters, a command letter followed by a speed (e.g., F150 to move forward at speed 150, L120 to turn left at speed 120, S for stop). The ESP32 translates these commands and communicates them to the Arduino using UART (hardware serial) on GPIO pins 16 and 17 at 9600 baud.

The ESP32 is given a static IP address (10.94.77.100) to ensure connection stability. The Python client also supports re-connection if the connection is lost.

VI. HARDWARE COMPONENTS

Table I lists the hardware components used in this system.

TABLE I

Hardware Components Summary

| Component | Model | Purpose |
| --- | --- | --- |
| Arduino Uno | ATmega328P | Motor control |
| ESP32 | ESP-32S | WiFi server |
| L298N | H-Bridge | Motor driver |
| 4WD Chassis | DC x4 | Movement |
| 18650 x2 | 3.7V each | Motor power |
| Power Bank | 5V USB | ESP32 power |
| Laptop+Cam | Python 3.12 | Detection |

VII. RESULTS AND DISCUSSION

The system was evaluated in an indoor scene with only one human in the scene. The following results were observed.

A. Human Detection

MediaPipe successfully detected the human pose and estimated the landmark locations at around 30 Hz. In most indoor lighting conditions, the system accurately detected the relative location of the person with respect to the camera.

B. Distance Control

The shoulder width had a good estimate of the distance to the person. The robot stopped moving when the person was too near (shoulder width > 200 px) and continued moving when the person was too far away (shoulder width < 100 px) to ensure a safe following distance.

C. PID Performance

The PID controller achieved smoother turns than an on-off controller. The robot made small corrections for small errors and large corrections for large errors, an effect that minimized oscillations.

D. Wireless Communication

The WiFi communication system was stable during the experiment using a mobile phone hotspot (2.4 GHz). The command transmission delay was very short. The smart send delay time of 0.3 seconds ensured the system was not overwhelmed and the camera feed did not lag.

VIII. CHALLENGES AND SOLUTIONS

There were some issues during development. The incompatibility between the MediaPipe library and Python 3.12 was solved by using version 0.10.13. The ESP32 WiFi connection issue was found to be due to spaces in the hotspot name and was solved by renaming the hotspot. The ESP32 voltage issue due to being powered from the 5V pin on the L298N was resolved by supplying power from a USB power bank. Camera delay due to frequent commands was solved by sending commands at least every 0.3 second. The failure of the motor to move was due to wiring at the L298N output pins.

IX. FUTURE IMPROVEMENTS

Several improvements are planned. Fitting the camera to the robot chassis would allow it to autonomously follow a target over long distances. Mounting ultrasonic sensors will allow object avoidance. Mounting a Raspberry Pi on the robot would allow it to be fully mobile. PID tuning with Ziegler-Nichols algorithm would also enhance controller performance. Person re-identification would enable tracking after brief loss of the target.

X. CONCLUSION

In this paper, we implemented a wireless PID controlled human following robot. The computer vision system using MediaPipe, the wireless communication using ESP32, motor control using Arduino and PID speed control allowed the robot to follow autonomously. It accurately locates humans, keeps an appropriate distance, and makes smooth turns. The system offers hands-on experience in computer vision, embedded systems, wireless communication and PID control, making it an excellent foundation for future robotics projects.

. REFERENCES

[1] Google LLC, “MediaPipe Solutions,” 2023. [Online]. Available: https://mediapipe.dev

[2] OpenCV Team, “OpenCV Library,” 2023. [Online]. Available: https://opencv.org

[3] Espressif Systems, “ESP32 Technical Reference Manual,” 2023.

[4] Arduino, “Arduino Uno Rev3 Datasheet,” 2023. [Online]. Available: https://arduino.cc

[5] K. Ogata, Modern Control Engineering, 5th ed. Prentice Hall, 2010.

[6] Python Software Foundation, “Python 3.12 Documentation,” 2023. [Online]. Available: https://python.org